Published: 27-Jun-2012

Fuel Charging and Controls - Turbocharger - ID4 2.4L Diesel - Turbocharger

Removal and Installation

Removal

|  | - WARNING: Do not work on or under a vehicle supported only by a jack. Always support the vehicle on safety stands. Raise and support the vehicle. For additional information, refer to: Lifting (100-02 Jacking and Lifting, Description and Operation). |
| --- | --- |

|  | - Remove the air cleaner outlet pipe. For additional information, refer to: Air Cleaner Outlet Pipe (303-12 Intake Air Distribution and Filtering - ID4 2.4L Diesel, Removal and Installation). |
| --- | --- |

|  | - Disconnect the charge air cooler inlet hose. - Release the clip. |
| --- | --- |

|  | - Disconnect the turbocharger electrical connector. |
| --- | --- |

|  | - Remove the turbocharger heat shield. - Remove the 5 bolts. |
| --- | --- |

|  | - Remove and discard the 4 nuts from the catalytic converter. |
| --- | --- |

|  | - Remove and discard the 4 studs from the catalytic converter. |
| --- | --- |

|  | - Release the catalytic converter. - Remove and discard the 2 nuts. |
| --- | --- |

|  | - Release the catalytic converter. - Loosen the 4 bolts. |
| --- | --- |

|  | - Remove the catalytic converter bracket. - Remove the bolt. |
| --- | --- |

|  | - CAUTION: Make sure that all openings are sealed. Use new blanking caps. Disconnect the turbocharger oil supply pipe. - Remove the banjo bolt. - Remove and discard the sealing washers. |
| --- | --- |

|  | - CAUTION: Make sure that all openings are sealed. Use new blanking caps. Release the turbocharger oil return tube. - Remove the 2 bolts. - Remove and discard the gasket. |
| --- | --- |

|  | - Remove and discard the 3 turbocharger nuts. |
| --- | --- |

|  | - CAUTION: Make sure that all openings are sealed. Use new blanking caps. Remove the turbocharger. - Remove and discard stud. - Remove and discard the gasket. |
| --- | --- |

|  | - NOTE: Do not disassemble further if the component is removed for access only. Remove the turbocharger inlet duct. - Remove the 2 bolts. |
| --- | --- |

Installation

|  | - Install the turbocharger inlet duct. - Tighten to 23 Nm (17 lb.ft). |
| --- | --- |

|  | - NOTE: Remove and discard the blanking caps. Install the turbocharger. - Install a new gasket. - Install new studs. |
| --- | --- |

|  | - Install new turbocharger nuts. - Tighten the nut to 23 Nm (17 lb.ft). |
| --- | --- |

|  | - NOTE: Remove and discard the blanking caps. Secure the turbocharger oil return tube. - Install a new gasket. - Tighten the bolts to 10 Nm (7 lb.ft). |
| --- | --- |

|  | - NOTE: Remove and discard the blanking caps. Connect the turbocharger oil supply pipe. - Install new sealing washers. - Tighten the banjo bolt to 35 Nm (26 lb.ft). |
| --- | --- |

|  | - Install the catalytic converter bracket. - Loosely install the bolt. |
| --- | --- |

|  | - Install 4 new studs to the catalytic converter. |
| --- | --- |

|  | - Install 4 new nuts to the catalytic converter. - Tighten to 45 Nm (33 lb.ft). |
| --- | --- |

|  | - Tighten to 30 Nm (22 lb.ft). |
| --- | --- |

|  | - Tighten to 30 Nm (22 lb.ft). |
| --- | --- |

|  | - Install new nuts. - Tighten to 30 Nm (22 lb.ft). |
| --- | --- |

|  | - Tighten to 20 Nm (15 lb.ft). |
| --- | --- |

|  | - Install the turbocharger heat shield. - Tighten to 10 Nm (7 lb.ft). |
| --- | --- |

|  | - Connect the turbocharger electrical connector. |
| --- | --- |

|  | - Tighten to 5 Nm (4 lb.ft). |
| --- | --- |

|  | - Install the air cleaner outlet pipe. For additional information, refer to: Air Cleaner Outlet Pipe (303-12 Intake Air Distribution and Filtering - ID4 2.4L Diesel, Removal and Installation). |
| --- | --- |